|  | Jacobian | Modal decomposition | Singular Value Decomposition |
| --- | --- | --- | --- |
| Philosophy | Topology-based decoupling | Physics-based decoupling | Data driven-based decoupling |
| Requirements | Accurate knowledge of the geometry and the frame of reference about which the Jacobians are evaluated | Knowledge of the physics of the plant and system parameters like mass, stiffness and damping matrices | Knowledge of the plant transfer function matrix, which can be obtained experimentally |
| Decomposition | Ja-1GJτ-1 | Cm-1GBm-1 | U-1GV-T |
| Method | Vector multiplication | Eigenvalue decomposition | Singular value decomposition (SVD) |
| Operation matrix | Frequency dependent complex plant transfer matrix | Frequency independent M-1K real matrix | Real-approximation of the complex plant matrix at the decoupling frequency |
| Matrix type | Complex rectangular/square | Real square | Real rectangular/square |
| Transformation vectors | Jacobians in actuator and sensor space | Eigenvectors | Singular vectors |
| Physical interpretation of the transformation vectors | Transformation of the velocity in the local frame to the fame in which Jacobian is evaluated | Directions of the different modes of the system | Directions of the maximum and minimum singular values |
| Decoupling | Centre of stiffness frame: Decoupling in the low frequency region | Decoupling in entire frequency band | Decoupling frequency can be chosen in the desired frequency band of interest |
|  | Centre of mass frame: Decoupling in high frequency region |  |  |
| Mathematical complexity for decomposition | (I don’t have any idea on this) | Ο(n3) | Ο((max(m, n)*min(m, n)) |
| Controller implementation | Jτ-1KdiagJa-1 | Bm-1KdiagCm-1 | V-TKdiagU-1 |
| Control effort for similar performance on Gravimeter model |  |  |  |
| Robustness |  |  |  |
| Applicability | Applicable for small motions (Since, Jacobian approximation is valid for small motion) | Applicable only to systems whose dynamics/governing equation can be expressed using mass, stiffness and damping matrices | Applicable to systems whose complete transfer matrix can be obtained experimentally (or numerically ) |